NDOT Procedural Guidelines

CLOSURES & TRUCK CLOSURES on I-80 W/B at Stateline:

I-80 may be closed to (prohibiting) Trucks only or to all Traffic.  Make sure you are posting the correct messages.

Overview: Beginning winter ‘07/’08, truck storage during I-80 truck closures is a big concern now that the

Boomtown truck-stop has closed.  Many flyers have been distributed to truck-stops etc. to get the word out. 	Regardless whether it is referred to as a truck “hold” or a “closure” field personnel are instructed to turn trucks 	around 	and not hold them unless under special circumstances, e.g. less than 15 minutes. (Caltrans refers to any 	 	closure < 2hrs as a hold & not a closure)

!!!!!!!   See ITS Guidelines for graduated levels:  D2 Levels   !!!!!!!!!

On all  Closures:

Call CalTrans Kingvale to confirm and get an estimate (their best guess if nothing else).

Notify: All Level 2 folks (both Maintenance Mgrs) except legal and safety

NHP (if they didn’t notify you)

Post:  (Trucks Only) “TURNING TRUCKS __X_ MILES AHEAD“

(or All Vehicles) “ I-80 CLOSED AT CA SATATELINE” / “CALL 511 FOR ROAD INFO”

on these W/B signs (X = distance to Stateline Exit 1): Humboldt (138), Trinity (84), Fernley (45),

*Mustang (24), *Vista (19), *Cemetery (11) & *Mogul (7)

* coordinate w/ Caltrans messages

Activate:  The flashers for “I-80 CLOSED TO TRUCKS AT STATELINE WHEN FLASHING” signs that are 			W/B before Trinity and Wadsworth exits. Code for ON is 8099. Code for OFF is 8000.

Email/TM: from AlertSenders; send a short message (<125 charac including subject) to the I-80 Truck Closure DL which is in the Outlook Global Address list. 	(the DL is a mix of both email and TM addresses)

Example: Subject: “I-80 Westbound is closed to Trucks at CA Stateline” Body: “Until at least XX:XXpm/am. 			Please do not reply.”

Additional measures for extended (estimated >3hrs) Truck-Closures as directed by 21-04 (Mike F.)

Typically on Sundays, under storm conditions, CalTrans will close to trucks from mid-morning to late evening, ie  	8-12 hr closure. For these and other extended truck-closures, the idea is get trucks to stop east of Reno or use 	alternate routes into California.

Estimated 3 - 6hr closure

Record a short Floodgate message (for 511 callers) and associate it with I-80 and US-395:

The California Dept. of Transportation has closed westbound I-80 to trucks at the CA state-line. All w/b trucks are 	being turned at 	Nevada exit 1 due to heavy traffic and storm conditions. Due to limited truck parking in the Reno 	area, please plan on parking further east of Reno, or plan on using an alternate route. This restriction will be in 	effect until further notice.  Check back for more info or visit the Nevada Dept. of Transportation’s website at 	www.nevadadot.com

Call Elko-Roads to post to Winnemucca and Elko DMSs:

“I-80 CLOSED TO TRUCKS AT CA STATE LINE, CALL 511 FOR ROAD INFO

OR if closed to all vehicles:

“I-80 CLOSED  AT CA STATE LINE, CALL 511 FOR ROAD INFO

Estimated 6 - 12hr closure

Record a short Floodgate message (for 511 callers) and associate it with I-80 and US-395:

The California Dept. of Transportation has closed westbound I-80 to trucks at the CA state-line. All w/b trucks are 	being turned at 	Nevada exit 1 due to heavy traffic and storm conditions. Due to limited truck parking in the Reno 	area, please plan on parking further east of Reno, or plan on using an alternate route. This restriction will be in 	effect until further notice.  Check back for more info or visit the Nevada Dept. of Transportation’s website at 	www.nevadadot.com

Call Elko-Roads to post to Wendover, Wells, Elko, Winnemucca & Wells DMSs:

“I-80 CLOSED TO TRUCKS AT CA STATE LINE, CALL 511 FOR ROAD INFO

OR if closed to all vehicles:

Record appropriate Floodgate (511) message

Post to DMS: “I-80 CLOSED  AT CA STATE LINE, CALL 511 FOR ROAD INFO”

Estimated >12hrs

This will generally require a manned truck screening at Trinity and Wadsworth

Call The Emergency Road Closure Coordinator (remind him to call you back w/ progress)

Note: Once volunteers are in place the Trinity & Wadsworth signs will manually be changed by them to read 			“ALL TRUCKS MUST USE NEXT EXIT”. Only local delivery up to Reno, mail, live animals & RR workers

will be allowed to proceed.

Record a short Floodgate message (for 511 callers) and associate it with I-80 and US-395:

The California Dept. of Transportation has closed westbound I-80 to trucks at the CA state-line. All w/b trucks are 	being turned at 	Nevada exit 1 due to heavy traffic and storm conditions. Due to limited truck parking in the Reno 	area, please plan on parking further east of Reno, or plan on using an alternate route. This restriction will be in 	effect until further notice.  Check back for more info or visit the Nevada Dept. of Transportation’s website at 	www.nevadadot.com

Call Elko-Roads to post to Winnemucca Elko, Wells & Wendover DMS’s:

“I-80 CLOSED TO TRUCKS AT CA STATE LINE USE ALT ROUTE, CALL 511 FOR ROAD INFO

OR if closed to all vehicles:

Record appropriate Floodgate (511) message. (511 callers hear the message after they choose I-80)

“I-80 CLOSED  AT CA STATE LINE USE ALT ROUTE, CALL 511 FOR ROAD INFO

Plans for portable HAR in Lovelock are underway. This would include a portable DMS in Lovelock advising trucks (or all motorists) to tune to an AM station for information. CalTrans would control the content of the HAR message. If trucks are not being screened east of Reno, it would broadcast same message as their Boomtown HAR. If trucks are being screened east of Reno call CalTrans with the appropriate message to broadcast. nov’08; portable DMS & portable HAR for Lovelock are out of service.

Utilizing  HAR; For large-scale events or extended closures, you should recorded a General Transportation 511 	Alert, which goes out to HAR as well as all 511 callers.

If you’ve posted a Floodgate, you may also want to record a HAR message and then post “TUNE TO AM530” 	(or AM1610) on DMS

Remember there are  3 types of  511 Alerts (AMBER, General Transportation & Homeland Security), use the 	General Transportation Alert for closures. 511 Alerts are announced to all 511 callers and also on HAR. 	Floodgate messages, on the other hand, require the 511 caller to select the associated route to hear the message 	and currently do not carry-over to HAR. Recording a HAR Announcement only broadcasts on HAR (not to 511 	phone

“The good thing about the 511 Alerts is that if we have a huge issue going on, callers can call in, and without 	making any selections, 	they can hear the critical info….then they might hang up and free up that line for 	another caller, which during a 	horrific event, could be important.” Denise Inda 5’10